Contents

1. Small to Scale Networks	1

2. Network	1

Definition:	1

Types of Networks (based on scale):	1

Key Terms:	1

3. Communication Network	2

4. Communication Patterns	2

5. Communication Model	2

6. Message Transmission Process	2

7. Important Networking Tasks	2

8. Transmission Modes	3

9. Switching Techniques (Do read notebook for this)	3

10. Network Topologies (Do read notebook for this)	3

11. Transmission Media	3

12. Network Devices	4

13. Collision & Broadcast Domains	4

14. Data Link Layer	4

15. Models	5

16. APDU, PPDU, SPDU	5

17. Headers and Trailers	5

18. Properties of Well-Known Protocols	6

5.	In-Band vs Out-of-Band Signaling	6

6.	Connection-Oriented vs Connectionless: Connection-oriented protocols establish a session before transmitting data, while connectionless protocols send data without setup.	7

19. Application Layer	7

20. Presentation Layer	8

21. Session Layer	9

22. Transport Layer	9

16. Addressing & Subnetting	11

17. Error Control (ARQ)	11

18. Performance Metrics	11

19. Security Basics	11

CCN NOTES (Till Week 4)

### 1. Small to Scale Networks

These are different types of computer networks:

- LAN (Local Area Network)

  - A network within a small area (like home, office, school).

- SAN (Storage Area Network)

  - A network that connects storage devices (like databases, servers).

- SDN (Software Defined Networking)

  - A modern type of networking where software controls and manages the network instead of only hardware.

- NFV (Network Function Virtualization)

  - Network services like firewalls, routers, etc., are provided using software instead of physical devices.

- Cloud Sim (Packet Tracer)

  - A simulation tool to practice and test network configurations.

### 2. Network

### Definition:

- A network is a collection of interconnected devices (nodes/hosts) that can communicate and share resources (data, files, printers, internet, applications).

### Types of Networks (based on scale):

- PAN (Personal Area Network): Very small (e.g., Bluetooth headset ↔ phone).

- LAN (Local Area Network): Within a building/campus (Ethernet, Wi-Fi).

- MAN (Metropolitan Area Network): Across a city (ISPs).

- WAN (Wide Area Network): Global, interconnects LANs/MANs (Internet).

### Key Terms:

- Node / Host: Device connected to the network (PC, mobile, server).

- Link / Medium: Communication channel (wired: Ethernet, fiber; wireless: radio).

- Network Topology: Physical or logical layout (bus, star, ring, mesh, hybrid).

### 3. Communication Network

- A system that allows communication between devices using hardware (nodes, links) and protocols.

- Examples: LANs, WANs, Internet, mobile networks.

### 4. Communication Patterns

- Unicast: One sender → one receiver.

- Broadcast: One sender → all receivers in network.

- Multicast: One sender → selected group of receivers.

- Anycast: One sender → nearest/best receiver out of many.

- Geocast: One sender → all receivers in a geographic region.

### 5. Communication Model

- Transmitter (Sender device): Creates and encodes message.

- Medium (Channel): Carries signals (wired/wireless). May introduce noise.

- Receiver (Device): Accepts signals and decodes them.

- Destination (End System): Final application/user receives original message.

Example: WhatsApp message → phone (transmitter) → Wi-Fi (medium) → server + friend’s phone (receiver) → WhatsApp app (destination).

### 6. Message Transmission Process

- Message created at application level.

- Converted to digital data, broken into packets.

- Passes through network devices (switches/routers).

- Transmitted via medium as signals.

- At receiver side, signals → packets → message reassembled.

### 7. Important Networking Tasks

- Routing Control: Selects best path for packets. Uses protocols (RIP, OSPF, BGP).

- Forwarding: Moves packet from input port → output port.

- Segmentation & Reassembly: Breaks big data into small packets, reassembles at receiver.

- Flow Control: Adjusts sender speed so receiver is not overloaded (TCP sliding window).

- Error Control: Detects/corrects errors (parity, checksum, CRC, ARQ methods).

- Congestion Control: Prevents overload in network (TCP slow start, fast retransmit).

- Multiplexing/Demultiplexing: Combines multiple signals on one medium, separates at receiver.

- Addressing: Assigns MAC and IP addresses to devices.

### 8. Transmission Modes

- Simplex: Data is sent in one direction only (e.g., keyboard → PC).

- Half-duplex: Data is sent in both directions, but one at a time (e.g., walkie-talkie).

- Full-duplex: Data is sent in both directions at same time (e.g., telephone).

### 9. Switching Techniques (Do read notebook for this)

- Circuit Switching: Dedicated path for whole session. Used in old telephony. Guaranteed QoS but wastes resources.

- Packet Switching: Data split into packets, each follows independent path. Used in Internet. Efficient but variable delay.

- Message Switching: Entire message stored and forwarded at each hop. Reliable but slow.

### 10. Network Topologies (Do read notebook for this)

- Bus: Single backbone cable; easy but failure breaks whole network.

- Star: Central hub/switch; easy to manage but central device is single point of failure.

- Ring: Devices connected in loop; each forwards data. Failure in one link can disrupt.

- Mesh: Every device connects to every other; reliable but costly.

- Hybrid: Combination of topologies (star + bus, etc.).

### 11. Transmission Media

- Guided (Wired):

  - Twisted Pair (Cat5/6): Cheap, common in LAN.

  - Coaxial Cable: Used in older networks, cable TV.

  - Optical Fiber: High speed, long distance, immune to EMI.

- Unguided (Wireless):

  - Radio waves: Wi-Fi, Bluetooth.

  - Microwaves: Long-distance, point-to-point.

  - Infrared: Short range, line-of-sight.

  - Satellite: Global coverage, high latency.

### 12. Network Devices

- Repeater: A repeater is a physical layer device that regenerates and amplifies weak or distorted signals to extend the transmission distance in a network. (Physical layer).

- Hub: A hub is a multiport repeater that broadcasts incoming signals to all connected devices, regardless of destination. It works at the physical layer and does not filter or manage traffic.

- Bridge: A bridge is a data link layer device that connects two LAN segments and filters traffic using MAC addresses, allowing or blocking frames based on destination MAC.

- Switch: A switch is a multiport bridge that forwards frames intelligently based on MAC address learning. It reduces collisions by creating separate collision domains for each port.

- Router: A router is a network layer device that forwards packets between different networks using logical (IP) addressing. It determines the best path using routing tables and protocols.

- Gateway: A gateway is a device that acts as a translator between networks using different protocols or architectures, enabling communication between otherwise incompatible systems.

- Modem: A modem (Modulator-Demodulator) converts digital signals from a computer into analog signals for transmission over telephone lines and vice versa.

### 13. Collision & Broadcast Domains

- Collision Domain: Area where data packets can collide. Reduced by switches.

- Broadcast Domain: Area where broadcast traffic reaches. Reduced by routers or VLANs.

### 14. Data Link Layer

- Frame: Header (MAC src/dst), Payload (data), Trailer (CRC).

- Error Detection: Parity bit, checksum, CRC.

- Access Control:

  - CSMA/CD (Ethernet): Detect collisions.

  - CSMA/CA (Wi-Fi): Avoid collisions.

  - Token Passing: Deterministic access (Token Ring).

### 15. Models

- OSI Model (Open Systems Interconnection Model):
Definition:
A conceptual 7-layer model that standardizes how systems communicate over a network. Each layer provides services to the layer above and uses services of the layer below.

Layers (top to bottom):

- Application Layer: Provides services and interfaces to applications (web, email, file transfer). Deals with APDUs.

- Presentation Layer: Ensures data is in a usable, consistent format. Handles encryption, compression, and translation. Deals with PPDUs.

- Session Layer: Establishes, manages, and terminates sessions. Provides synchronization and dialog control. Deals with SPDUs.

- Transport Layer: Ensures reliable end-to-end delivery, error control, flow control, and segmentation. PDU = Segment.

- Network Layer: Provides logical addressing and routing (IP). PDU = Packet.

- Data Link Layer: Provides physical addressing (MAC), framing, and error detection. PDU = Frame.

- Physical Layer: Transmits raw bits over medium (cables, signals). PDU = Bits.

- TCP/IP Model (4 layers):
Link → Internet → Transport → Application.

- Protocols:

  - HTTP/HTTPS (web).

  - TCP/UDP (transport).

  - IP (network).

  - ARP, DHCP, DNS (support).

### 16. APDU, PPDU, SPDU

- APDU (Application Protocol Data Unit): Data unit at the Application layer (e.g., an HTTP request or SMTP message).

- PPDU (Presentation PDU): Data after being formatted, compressed, or encrypted by Presentation layer.

- SPDU (Session PDU): Session-control information (synchronization points, dialog tokens) added at Session layer.

### 17. Headers and Trailers

Definition:
Extra control information added to the beginning (header) or end (trailer) of data at each layer during encapsulation.

- Header: Contains source/destination addresses, sequence numbers, protocol identifiers, flags, etc.

- Trailer: Often used for error detection (e.g., CRC in Ethernet frames).

Example of Encapsulation:
Application data → add Transport header (ports) → add Network header (IP addresses) → add Data Link header + trailer (MAC + CRC) → converted into bits at Physical layer.

### 18. Properties of Well-Known Protocols

A good protocol is defined by:

- Reliability: Reliability means whether the protocol guarantees delivery of data without errors, duplication, or loss.

- Example: TCP is reliable (ACKs, retransmission).

- Example: UDP is unreliable (no delivery guarantee).

- State: A stateful protocol maintains context of a session, while a stateless protocol treats each request independently.

- Example: FTP, TCP are stateful.

- Example: HTTP is stateless.

- Efficiency: Refers to protocol overhead vs usable data.

- Example: UDP has low overhead → efficient but less safe.

- Example: TCP has more overhead → less efficient but reliable.

- Security: Whether the protocol provides encryption, authentication, and integrity.

- Example: HTTPS, SSH are secure.

- Example: HTTP, FTP (without extensions) are insecure.

### In-Band vs Out-of-Band Signaling

- In-Band: Control information (signaling) is sent along the same channel as data.

  - Example: FTP uses in-band commands on the control channel.

- Out-of-Band: Control information is sent on a separate dedicated channel.

  - Example: HTTP uses a separate control (headers) and data (body) but still within same TCP connection → sometimes considered “in-band.”

  - Example: FTP’s control vs data channel is a clearer demonstration of out-of-band.

### Connection-Oriented vs Connectionless: Connection-oriented protocols establish a session before transmitting data, while connectionless protocols send data without setup.

- Example: TCP is connection-oriented.

- Example: UDP is connectionless.

### 19. Application Layer

Definition:
Topmost OSI layer that provides services for user applications and enables network-based communication.

Common Protocols:

#### HTTP (HyperText Transfer Protocol)

- Definition: Protocol for communication between web browsers and servers.

- Transport: TCP. Port: 80 (HTTP), 443 (HTTPS = HTTP over TLS).

- Properties: Stateless, request/response model. Uses methods like GET, POST, PUT, DELETE.

- Messages: Request line + headers + body (optional). Response line + headers + body.

#### FTP (File Transfer Protocol)

- Definition: Transfers files between client and server.

- Transport: TCP. Ports: 21 (control), 20 (data).

- Properties: Stateful, uses two channels: control connection for commands, data connection for file transfers. Supports active and passive modes.

#### DHCP (Dynamic Host Configuration Protocol)

- Definition: Assigns IP addresses and network parameters automatically to clients.

- Transport: UDP. Ports: 67 (server), 68 (client).

- Process: DORA (Discover, Offer, Request, Acknowledge).

- Properties: Broadcast-based, leases IPs for limited time.

#### SMTP (Simple Mail Transfer Protocol)

- Definition: Sends and forwards emails between servers.

- Transport: TCP. Ports: 25 (server-to-server), 587 (submission).

- Commands: HELO/EHLO, MAIL FROM, RCPT TO, DATA, QUIT.

- Properties: Reliable, text-based, store-and-forward.

#### POP3 (Post Office Protocol v3)

- Definition: Retrieves emails from a mail server to a local client.

- Transport: TCP. Ports: 110 (plain), 995 (secure POP3S).

- Properties: Simple, downloads emails and often deletes from server.

#### MIME (Multipurpose Internet Mail Extensions)

- Definition: Extends email to support attachments, multimedia, non-ASCII text.

- Properties: Uses Content-Type, Content-Encoding, Content-Disposition headers. Encodes files (Base64, quoted-printable).

#### SSH (Secure Shell)

- Definition: Secure protocol for remote login and command execution.

- Transport: TCP. Port: 22.

- Properties: Provides encryption, authentication, integrity. Supports tunneling, SCP/SFTP.

#### DNS (Domain Name System)

- Definition: Resolves domain names into IP addresses.

- Transport: UDP (queries), TCP (zone transfers, large replies). Port: 53.

- Message Structure: Header (ID, flags, counts), Question (query), Answer, Authority, Additional.

- Records: A (IPv4), AAAA (IPv6), MX (mail), CNAME (alias), NS (name server), PTR (reverse lookup).

### 20. Presentation Layer

Definition:
Ensures that data is in a form usable by applications. Translates between application and network formats.

Functions:

- Data format translation (ASCII ↔ EBCDIC, big-endian ↔ little-endian).

- Compression (reduces data size).

- Encryption/Decryption (TLS/SSL).

- Encoding of structured data (ASN.1, XML, JSON).

### 21. Session Layer

Definition:
Manages sessions (continuous exchanges) between applications.

Functions:

- Establish, maintain, and terminate sessions.

- Synchronization using checkpoints (so transfer can resume).

- Dialog control (who can send, full/half duplex).

- Token management (prevent collisions in multi-user systems).

### 22. Transport Layer

Definition:
Provides reliable process-to-process communication. Ensures correct delivery, sequencing, flow control, and error handling.

Two main protocols:

#### TCP (Transmission Control Protocol)

- Definition: Connection-oriented, reliable, byte-stream service.

Fields explained:

- Source Port (16 bits) — sending application’s port.

- Destination Port (16 bits) — receiving application’s port.

- Sequence Number (32 bits) — byte-stream position of the first byte in this segment (used for ordering & retransmit).

- Acknowledgment Number (32 bits) — next sequence number expected by sender of this ACK (valid only if ACK flag is set).

- Data Offset (4 bits) — header length in 32-bit words (minimum 5 => 20 bytes). Indicates where data begins.

- Reserved (6 bits historically) — reserved for future use (should be zero).

- Control Flags (6 bits classic):

  - URG — Urgent pointer field is valid (urgent data).

  - ACK — Acknowledgment field is significant.

  - PSH — Push: deliver data to application immediately.

  - RST — Reset connection.

  - SYN — Synchronize sequence numbers (used to open connection).

  - FIN — No more data from sender (close).
(Modern extensions add flags like CWR, ECE, NS — used for ECN and other extensions.)

- Window Size (16 bits) — flow-control window: number of bytes receiver is willing to accept starting from ack number. Can be scaled using Window Scale option in TCP options.

- Checksum (16 bits) — covers TCP header + data + pseudo-header (src IP, dst IP, protocol, TCP length) to protect against corruption.

- Urgent Pointer (16 bits) — if URG set, gives offset from sequence number to urgent data end.

- Options (variable): optional extra control:

  - MSS (Maximum Segment Size) — negotiation of largest payload allowed.

  - Window Scale — allows large windows by scaling factor.

  - SACK Permitted / SACK blocks — selective acknowledgement to retransmit only lost segments.

  - Timestamps (PAWS) — RTT calculation & protection against old segments.

- Padding: to ensure header ends on 32-bit boundary.

Important TCP behaviors (brief):

- 3-way handshake (establish): Client → SYN (seq=x) ; Server → SYN+ACK (seq=y, ack=x+1) ; Client → ACK (ack=y+1) → connection established.

- 4-step close: one side sends FIN → other replies ACK, then sends FIN → first replies ACK. (Half-close possible.)

- Reliability: ACK-based retransmission (retransmit on timeout or fast retransmit via duplicate ACKs).

- Flow control: sliding window using Window field; receiver advertises window.

- Congestion control: slow start, congestion avoidance, fast retransmit, fast recovery (algorithms evolved: Tahoe/Reno/CUBIC).

- Checksum: mandatory; detects corrupted segments.

- Ports: used for demux; server uses well-known ports.

#### UDP (User Datagram Protocol)

- Definition: Connectionless, lightweight protocol for fast communication.

- Properties: No reliability, no ordering, no congestion control, minimal overhead.

- Header (8 bytes):

Fields explained:

- Source Port (16) — optional, used for reply demux.

- Destination Port (16) — application port at receiver.

- Length (16) — length of UDP header + data in bytes.

- Checksum (16) — covers UDP header + data + pseudo-header (IP src/dst/proto/length). In IPv4, checksum is optional (0 = no checksum) but recommended; in IPv6, checksum is mandatory.

### 16. Addressing & Subnetting

- MAC Address: 48-bit hardware address.

- IP Address (IPv4): 32-bit logical address (e.g., 192.168.1.1/24).

- Subnetting: Divides large network into smaller.
Example: 192.168.1.0/24 → four /26 subnets (64 hosts each).

- Special Protocols: ARP (IP→MAC), DHCP (dynamic IPs), DNS (names→IPs), NAT (private→public).

### 17. Error Control (ARQ)

- Stop-and-Wait: One frame → ACK → next frame. Simple but inefficient.

- Go-Back-N: Sliding window, retransmit from error frame onward.

- Selective Repeat: Only retransmit erroneous frames. Efficient but complex.

### 18. Performance Metrics

- Bandwidth: Max link capacity (bps).

- Throughput: Actual achieved data rate.

- Latency: Transmission + Propagation + Processing + Queuing delays.

- RTT: Round-trip time (source → destination → back).

- Jitter: Variation in delay.

- Packet Loss: Packets dropped in network.

### 19. Security Basics

- Confidentiality: Encryption (TLS, AES).

- Integrity: Hashing (SHA, MD5).

- Authentication: Passwords, certificates.

- Firewalls: Filters traffic.

- VPNs: Encrypts communication across public Internet.